PROTOCOL
to laboratory exercise

Negative Feedback

| Group / Class | Secretary | Signature |
| --- | --- | --- |
| 5 / 4BHELS | HOFSTÄTTER A. |  |
| Exercise- / Delivery date | Employee | Signature |
| 16. Dec. 2014 17. Dec. 2014 | BIEHL S. |  |
| Teacher | Employee | Signature |
| Tillich |  |  |
| Grade | Employee | Signature |
| Negative Feedback BC546 & BC556 |  |  |
| Used Devices Nr.; Device; Manufactor; Type; Place Nr.. 1.; Function Generator; HAMEG; HM8030-6; -. 2.; Power Supply; Conrad; PS2403D; -. 3.; Oscilloscope; Tektronix; TDS 1001B; -. Used Programs Nr.; Name; Version. 1.; Altium Designer; 13. 2.; Micro-Cap; 11. |

ÜBUNGS-/ABGABE-DATUM

Klasse /Gruppe

NOTE

LEHRER

# Table of Contents

1	Table of Contents	2

2	Tasks	3

2.1	Measurement Circuit	3

2.2	Given values	3

2.2.1	Assumptions	3

3	Calculation and Dimensioning	4

3.1	Needed Voltages	4

3.2	Resistors	4

3.3	Capacitors	4

4	Basic Measurements	5

4.1	Bias Voltages	5

4.2	Gain voltage	5

4.3	Voltage Capability	6

5	Bode Plot	8

5.1	Measured Values	8

5.2	Amplitude response (jω)	9

5.3	Phase response	9

6	Input and output resistance rIN and rOUT	10

6.1	Input resistance rIN	10

6.1.1	Measurement setup	10

6.1.2	Measurement results	10

6.2	Output resistance rOUT	11

6.2.1	Measurement setup	11

6.2.1	Measurement results	11

7	Open loop gain	12

7.1	Measurement circuit	12

7.2	Measurement results	12

8	Simulation	13

8.1	Voltages and currents (Bias)	13

8.1.1	Voltages	13

8.1.2	Currents	13

8.2	Bode plot	14

8.2.1	Simulation Circuit	14

8.2.2	Simulation Setup	14

8.2.3	Simulation Results	15

9	List of Figures	16

# Tasks

Task of this laboratory exercise was to build a Two Stage Complementary Amplifier. This circuit is used to amplify the voltage based on the given gain.

- Calculation, construction

- Measurement of the BIAS voltage

- Measurement of the gain and voltage capability.

- Measurement and visualizing of the Bode Plot (Amplitude- and Phaseresponse)

- Measuring of input and output resistance (rIN and rOUT)

- Measurement of the Open Loop Gain

## Measurement Circuit

Figure 1 - Circuit of a two stage complementary amplifier

## Given values

The taken assumptions at point 2.2.1 Assumptions were taken based on the following dependencies.

1 V≤UE≤2 V

UCE2≈URC2

100 µA≤IC1,IC2≤1 mA

10*IB1≤IQ≤20*IB2

### Assumptions

| IC1=IC2=500 µA | Vcc=15 V | fg=100 Hz | B=β=300 |
| --- | --- | --- | --- |
| UE1=1.5 V | UB1=UB2=2.1 V | RA=10 kΩ | Vu=30=29,54 dB |

Table 1 - Value Assumptions

# Calculation and Dimensioning

IB1=IC1250=500 µA250=2 µA

IQ=20*IB1=20*2 µA=40 µA

## Needed Voltages

UCE2=URC2=VCC- UE12=15 V-1.5 V2=6.75 V

URB2= VCC-UB1=15-2.1 V=12.9 V

## Resistors

Therefore that all needed voltages were assumed or calculated all resistors could be calculated now.

RC1=URC1IC1=600 mV500 µA=1.2 kΩ

RC2=URC2IC2=6.75 V500 µA=15 kΩ

RB1= URB1IQ=2.24 V40 µA=56 kΩ

RB2=URB2IQ=12.9 V40 µA=330 kΩ

RE1= UE1IC1+ IC2=1.5 V1 mA=1.5 kΩ

The formula for calculating the gain was converted to calculate RE2, the last needed resistor.

RE2= -RC2* RE1RC2-Vu*RE1=750 Ω →RE2=560 Ω

To get a more accurate gain the resistor RE2 were chosen to be a smaller one, this would make a definite amplification of 30.

## Capacitors

For the following calculations the corner frequency was 100 Hz. Depending on real and available component values the next higher capacitor was choosen.

C1= 12π* fg*(RB1//RB2)=12π* 100 Hz*(56  kΩ//330  kΩ)=33.24 nF→C1=33 nF

C2= 12π*fg* RE2=12π*100 Hz* 560 Ω =2,84 µF→C2=10 µF

C3= 12π* fg* RA=12π* 100 Hz*10 kΩ=160 nF→C3=220 nF

# Basic Measurements

## Bias Voltages

After final construction of the amplifier the bias voltages were measured. In this case the amplifier was operated with no input signal, so the operating point could be set and the bias voltages measured probably.

|  | Calculated | Measured |
| --- | --- | --- |
| UB1 | 2.1 V | 2 V |
| UE1 | 1.5 V | 1.5 V |
| UBE | 0.6 V | 0.6 V |
| UC1 | 0.6 V | 0.6 V |
| UBE2 | 0.6 V | 0.6 V |
| UCE1 | 6.75 V | 6.46 V |
| URC2 | 6.75V | 7.19 V |
| URB1 | 2.24 V | 2.13 V |
| URB2 | 12.9 V | 12.99 V |

Table 2 - Calculated and measured Values of the bias voltages

As seen in above table the measured and calculated values are very similar. Some differences were noticed but these are based on parasitic errors and irregular and non-ideal components.

## Gain voltage

Figure 2 - Gain voltage

The amplifier was operated with an amplitude of 208 mVpp, the measured output voltage was 5.80 V.

That implies that the gain is as assumed about 30.

Vu= UAUE= 5,8 V208 mV=27,88=28,90 dB

In the gain voltage scope an amplification of 27.8 can be seen instead of 30. A more accurate measurement result would be achieved with a lower RE2. This would cause a bigger amplification.

## Voltage Capability

The voltage capability Uemax is about 225mV. Is the input voltage higher than this voltage the amplifier starts going in saturation.

Figure 3 - Maximal voltage capability

With a higher input voltage (≥480 mV) the output signal is already distorted. As seen in the following picture an amplitude of 229 mVpp leads already to a saturation.

Figure 4 - Slightly distorted amplifier

With an amplitude of about 1 Vpp, which is way too high for a normal and functional operation, the amplifier is heavy overmodulated and already on saturation.

With 1 Vpp there is already a very steep rising rectangle signal at the output.

Figure 5 - Heavy distorted amplifier

# Bode Plot

The following three values (Ue, Ua and dt) were measured with an oscilloscope by several multiple frequencies (20, 50, 100, 500, 1 000, 5 000, 10 000, 50 000, 1 000 000, 2 500 000 and 5 000 000) Hz

## Measured Values

| f [Hz] | UE [V] | UA [V] | jω [dB] | dt [ms] | phi [°] |
| --- | --- | --- | --- | --- | --- |
| 20 | 0.181 | 0.7 | 11.7483893 | 12.08333333 | 87 |
| 50 | 0.181 | 1.8 | 19.9518786 | 2.2 | 39.6 |
| 100 | 0.181 | 3.38 | 25.4247625 | 0.27 | 9.72 |
| 500 | 0.181 | 5.2 | 29.1664954 | 0.022 | 3.96 |
| 1,000 | 0.181 | 5.3 | 29.3319459 | 0.0084 | 3.024 |
| 5,000 | 0.181 | 5.22 | 29.1998386 | 0.002 | 3.6 |
| 10,000 | 0.181 | 5.06 | 28.9294388 | 0.0008 | 2.88 |
| 5,000 | 0.181 | 5.02 | 28.8605028 | 0.0000534 | 0.9612 |
| 1,000,000 | 0.181 | 4.8 | 28.4712533 | 0 | 0 |
| 2,500,000 | 0.181 | 2.9 | 24.0943885 | -0.000024 | -21.6 |
| 5,000,000 | 0.181 | 1.7 | 19.4554069 | -0.000046 | -82.8 |

Table 3 - Bode plot measurement values

## Amplitude response (jω)

Figure 6 - Bode plot amplitude response

## Phase response

Figure 7 - Bode plot phase response

# Input and output resistance rIN and rOUT

To measure the resistances of the amplifier, the input and output resistance, an additional extra circuit was built.

## Input resistance rIN

### Measurement setup

Figure 8 - Measurement circuit for the input resistance

### Measurement results

The circuit was operated with a functiongenerator and the parameters of 200 mV and a frequency of 10 kHz at the input of the amplifier.

The 100 kΩ Potentiometer was changed until the measured voltage behind the capacitor UE was half of the voltage from the input signal. UE was measured behind the capacitor because otherwise a phase shift would exist.

Figure 9 - Point where both voltages are the same

At the point where this happened (Ue≅108 mV) the finely adjusted resistance of the potentiometer had exactly the same value as the input resistance of the amplifier. The potentiometer had a measured resistance of 55 kΩ. Therefore the following assertion for rIN was made.

rIN=55 kΩ

The so measrured value could be calculated approximately with rIN≈56 kΩ // 330 kΩ≈47.87 kΩ.

The same measurement with an input frequency of 100 kHz resulted only in higher divergences. At 100 kHz the result for rIN was 62.4 kΩ which was much more inaccurate.

## Output resistance rOUT

### Measurement setup

Figure 10 - Measurement circuit for the output resistance

### Measurement results

The circuit was operated with a functiongenerator and the parameters of 10 V and a frequency of 10 kHz at the output of the amplifier. The capacitor located at the input was shunt to ground.

The idea behind this measurement setup was to send a signal at the output into the amplifier. The voltage between output and load resistor was measured and for these voltage the following formula was effected and applicable. The measured value for UA was 92 mV.

rOUT=RL*UAUin=10 kΩ*92 mV1 V→rOUT=920 Ω

# Open loop gain

To measure the open loop gain the circuit was a little bit adopted. First a voltage divider at the input was built, to make a very low input voltage (about 1 mV) possible. The voltage divider has a factor of 1:100 and therefore a 100 mV signal from the functiongenerator would affect an input signal of 1 mV at the amplifier input pin.

In addition to this the resistor RE2 was removed and the capacitor C2 replaced with a 100 µF Capacity.

With this modification the open loop gate could be measured with the voltage values of UE and UA.

## Measurement circuit

Figure 11 - Measurement circuit for the open loop gain

## Measurement results

Channel 1 was the voltage directly from the functiongenerator and the probe on channel 2 was connected to the amplifier output pin.

Figure 12 - Gain masurement for open loop

For correct calculations for the open loop gain the direct input voltage (behind the voltage divider) needs to be taken.

VOL=UAUE=3.64 V1.04 mV→VOL=3500≅70 dB

# Simulation

To simulate aboves already built circuit the same circuit was simulated in Micro-Cap to compare the simulations results with the measurement results.

## Voltages and currents (Bias)

### Voltages

Figure 13 - Simulation of the bias voltages

### Currents

Figure 14 - Simulation of the bias currents

## Bode plot

### Simulation Circuit

Figure 15 - Simulation circuit of the bode plot

### Simulation Setup

Figure 16 - Bode plot simulation settings

### Simulation Results

Figure 17 - Simulated bode plot in Micro-Cap

Finally can be seen that all realised simulations very similar if not even equal to the measured and calculated values were.

# List of Figures

Figure 1 - Circuit of a two stage complementary amplifier	3

Figure 2 - Gain voltage	5

Figure 3 - Maximal voltage capability	6

Figure 4 - Slightly distorted amplifier	6

Figure 5 - Heavy distorted amplifier	7

Figure 6 - Bode plot amplitude response	9

Figure 7 - Bode plot phase response	9

Figure 8 - Measurement circuit for the input resistance	10

Figure 9 - Point where both voltages are the same	10

Figure 10 - Measurement circuit for the output resistance	11

Figure 11 - Measurement circuit for the open loop gain	12

Figure 12 - Gain masurement for open loop	12

Figure 13 - Simulation of the bias voltages	13

Figure 14 - Simulation of the bias currents	13

Figure 15 - Simulation circuit of the bode plot	14

Figure 16 - Bode plot simulation settings	14

Figure 17 - Simulated bode plot in Micro-Cap	15